875—62.5(88A) Maintenance of covered equipment—requirements. An operator will conduct periodic inspections, repairs, tests, and maintenance as set forth in this rule and the manufacturer’s recommendations, as applicable. An operator will make a written record of all inspections, maintenance, tests, and repairs of covered equipment, and the records will be available to the director.

62.5(1) Nondestructive testing. The operator will ensure that appropriate nondestructive testing (NDT) is conducted and that documentation is available for review. NDT will be performed at the following times:

a.  At intervals recommended by the manufacturer;

b.  When required by the director due to a welded repair;

c.  When required by the director due to a visual indication of a potentially hazardous condition; and

d.  When recommended by a bulletin prepared according to ASTM F1193-14.

62.5(2) Wood components. The operator will remove a sufficient amount of soil around piling or wood members embedded in dirt to check for deterioration. When a wood piling requires replacement, the operator will install a concrete pier. The top of the pier will be installed so that the attached wood member is not exposed to dirt or water accumulation.

[ARC 8828C, IAB 1/22/25, effective 2/26/25]