761—511.9(321,321E) Annual all-systems oversize permits. Annual all-systems oversize permits are issued by the motor vehicle division for indivisible vehicles or indivisible loads for travel on the primary road system and specified city streets and county roads when the dimensions of the vehicle or load exceed statutory limits but the weight is within statutory limits. Routing is subject to embargoed bridges and roads and posted speed limits. The department will provide a map of the authorized city streets and county roads. Permit holders shall check the county 511 map available online at www.iceasb.org/roads and consult with local officials when traveling on county roads or city streets for bridge embargo, vertical clearance, detour, and road construction information. These permits are issued for the following:

511.9(1) Vehicles with indivisible loads, including special mobile equipment, mobile homes and factory-built structures, provided the following are not exceeded.

a.  Width. 12 feet 5 inches including appurtenances.

b.  Length. 120 feet 0 inches overall.

c.  Height. 13 feet 10 inches.

d.  Weight. Information contained in rule 761—511.17(321,321E).

e.  Distance. Movement is allowed for unlimited distance; routing through the motor vehicle division and city and county jurisdictions is not required.

511.9(2) Vehicles with indivisible loads, including special mobile equipment, mobile homes and factory-built structures, provided the following are not exceeded.

a.  Width. 14 feet 6 inches.

b.  Length. 120 feet 0 inches overall.

c.  Height. 15 feet 5 inches.

d.  Weight. Information contained in rule 761—511.17(321,321E).

e.  Distance. Movement is restricted to 50 miles unless trip routes are obtained from the motor vehicle division and city and county jurisdictions or the route continues on at least four-lane roads. Trip routes are valid for five days.

511.9(3) Vehicles with indivisible loads, including special mobile equipment, mobile homes and factory-built structures, provided the following are not exceeded.

a.  Width. 16 feet 0 inches.

b.  Length. 120 feet 0 inches.

c.  Height. 15 feet 5 inches.

d.  Weight. Information contained in rule 761—511.17(321,321E).

e.  Distance. Trip routes must be obtained from the motor vehicle division and city and county jurisdictions.

511.9(4) Truck trailers manufactured or assembled in the state of Iowa provided the following are met.

a.  Width. Not to exceed 10 feet 0 inches.

b.  Length. Overall combination length complies with Iowa Code section 321.457.

c.  Height. Statutory: Not to exceed 13 feet 6 inches.

d.  Weight. Information contained in rule 761—511.17(321,321E).

e.  Roadway width. At least 24 feet 0 inches.

f.  Limited movement. Only movement limited to the purpose of delivery or transfer from the point of manufacture or assembly to another point of manufacture or assembly within the state or to a point outside the state and on the most direct route necessary for the movement is allowed.

511.9(5) Necessary trip routes must be obtained from the appropriate city and county jurisdictions.

This rule is intended to implement Iowa Code sections 321.454, 321.456, 321.457, 321.463, 321E.2, 321E.3, 321E.8, 321E.10 and 321E.15.

[ARC 9512C, IAB 8/20/25, effective 9/24/25; ARC 0327D, IAB 6/10/26, effective 7/15/26]