481—1065.4(544A) Exceptions matrix. The following matrix is compiled to illustrate the exceptions outlined in Iowa Code section 544A.18. The laws and rules governing the practice of engineering are not illustrated herein.

| BUILDINGS NEW CONSTRUCTION |  |  |  |
| --- | --- | --- | --- |
| Building Use Type | Description | Architect Required | Architect May Not Be Required |
| Agricultural use | Including grain elevators and feed mills |  | X |
| Churches and accessory buildings whether attached or separate | One or two stories in height, up to a maximum of 2,000 square feet in gross floor area |  | X |
|  | Any number of stories in height, greater than 2,000 square feet in gross floor area | X |  |
|  | More than two stories in height | X |  |
| Commercial use | One story in height, up to a maximum of 10,000 square feet in gross floor area |  | X |
|  | One story in height, greater than 10,000 square feet in gross floor area | X |  |
|  | Two stories in height, up to a maximum of 6,000 square feet in gross floor area |  | X |
|  | Two stories in height, greater than 6,000 square feet of gross floor area | X |  |
|  | More than two stories in height | X |  |
| Detached residential use | One, two or three stories in height, containing 12 or fewer family dwelling units |  | X |
|  | More than 12 family dwelling units | X |  |
|  | More than three stories in height | X |  |
|  | Outbuildings in connection with detached residential buildings |  | X |
| Educational use |  | X |  |
| Hazardous use |  | X |  |
| Industrial use |  | X |  |
| Institutional use |  | X |  |
| Light industrial use |  |  | X |
| Places of assembly |  | X |  |
| Warehouse use | One story in height, up to a maximum of 10,000 square feet in gross floor area |  | X |
|  | One story in height, greater than 10,000 square feet in gross floor area | X |  |
|  | More than one story in height | X |  |
| Factory-built buildings | Any height and size, if certified by a professional engineer licensed under Iowa Code chapter 542B |  | X |
|  | One or two stories in height, up to a maximum of 20,000 square feet in gross floor area |  | X |
|  | One or two stories in height, greater than 20,000 square feet in gross floor area | X |  |
|  | More than two stories in height | X |  |
|  | More than 20,000 square feet in gross floor area | X |  |

| ALTERATIONS TO EXISTING BUILDINGS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Alteration Type | Description |  | Architect Required | Architect May Not Be Required |
| Structural alterations to exempt buildings | Modifications that change the structural members, means of egress, handicap accessible path, fire resistivity or other life safety concerns |  |  | X |
| Structural alterations to nonexempt buildings | Modifications that change the structural members, means of egress, handicap accessible path, fire resistivity or other life safety concerns |  | X |  |
| Nonstructural alteration | That does not modify means of egress, handicap accessible path, fire resistivity or other life safety concerns |  |  | X |
|  | That maintains the previous type of use |  |  | X |
| Nonstructural | A place of assembly of people or public gathering |  | X |  |
| alteration that | Educational use |  | X |  |
| changes the | Hazardous use |  | X |  |
| use of the building from any other use to: | Residential use exempted | and is one, two or three stories in height and contains not more than 12 family dwelling units |  | X |
|  | Residential use not exempted otherwise | and is more than three stories in height | X |  |
|  |  | and containing more than 12 family dwelling units | X |  |
| Nonstructural alterations that change the use of the building from industrial or warehouse to: | Commercial or office use | and is one story in height and not greater than a maximum of 10,000 square feet in gross floor area |  | X |
|  |  | and is one story in height and greater than 10,000 square feet in gross floor area | X |  |
|  |  | and is two stories in height and not greater than a maximum of 6,000 square feet in gross floor area |  | X |
|  |  | and is two stories in height and greater than 6,000 square feet in gross floor area | X |  |
|  |  | and is more than two stories in height | X |  |
|  |  | and is greater than 10,000 square feet of gross floor area | X |  |
| Nonstructural alterations to: | Agricultural use | Including grain elevators and feed mills |  | X |
|  | Churches and accessory building uses | One or two stories in height, up to a maximum of 2,000 square feet in gross floor area |  | X |
|  |  | Any number of stories in height, greater than 2,000 square feet in gross floor area | X |  |
|  |  | More than two stories in height | X |  |
|  | Commercial use | One story in height, up to a maximum of 10,000 square feet in gross floor area |  | X |
|  |  | One story in height, greater than 10,000 square feet in gross floor area | X |  |
|  |  | Two stories in height, up to a maximum of 6,000 square feet in gross floor area |  | X |
|  |  | Two stories in height, greater than 6,000 square feet in gross floor area | X |  |
|  |  | More than two stories in height | X |  |
|  | Detached residential buildings | One, two or three stories in height, containing 12 or fewer family dwelling units |  | X |
|  |  | More than 12 family dwelling units | X |  |
|  |  | More than three stories in height | X |  |
|  |  | Outbuildings in connection with detached residential buildings |  | X |
|  | Educational use |  | X |  |
|  | Hazardous use |  | X |  |
|  | Industrial use |  | X |  |
|  | Institutional use |  | X |  |
|  | Light industrial use |  |  | X |
|  | Places of assembly |  | X |  |
|  | Warehouse use | One story in height, up to a maximum of 10,000 square feet in gross floor area |  | X |
|  |  | One story in height, greater than 10,000 square feet in gross floor area | X |  |
|  |  | More than one story in height | X |  |
|  | Factory-built buildings | Any height and size if entire building is certified by a professional engineer licensed under Iowa Code chapter 542B |  | X |
|  |  | One or two stories in height, up to a maximum of 20,000 square feet of gross floor area |  | X |
|  |  | One or two stories in height, greater than 20,000 square feet in gross floor area | X |  |
|  |  | More than two stories in height | X |  |
|  |  | More than 20,000 square feet in gross floor area | X |  |

[ARC 7760C, IAB 4/17/24, effective 5/22/24; Editorial change: IAC Supplement 6/24/26]