641—37.5(136C) Units of exposure and dose.

37.5(1) As used in these rules, the quality factors for converting absorbed dose to dose equivalent are shown in Table I.

TABLE I

QUALITY FACTORS AND ABSORBED DOSE EQUIVALENCIES

| TYPE OF RADIATION | Quality Factor (Q) | Absorbed Dose Equal to a Unit Dose Equivalent1 |
| --- | --- | --- |
| X, gamma, or beta radiation and high-speed electrons | 1 | 1 |
| Alpha particles, multiple-charged particles, fission fragments and heavy particles of unknown charge | 20 | 0.05 |
| Neutrons of unknown energy | 10 | 0.1 |
| High-energy protons | 10 | 0.1 |

1. Absorbed dose in rad equal to 1 rem or the absorbed dose in gray equal to 1 sievert.

a.  If it is more convenient to measure the neutron fluence rate than to determine the neutron dose equivalent rate in sievert per hour or rem per hour, as provided in Table 1 above, 1 rem (0.01 Sv) of neutron radiation of unknown energies may, for purposes of these rules, be assumed to result from a total fluence of 25 million neutrons per square centimeter incident upon the body. If sufficient information exists to estimate the approximate energy distribution of the neutrons, the licensee or registrant may use the fluence rate per unit dose equivalent or the appropriate Q value from Table II to convert a measured tissue dose in gray or rad to dose equivalent in sievert or rem.

TABLE II

MEAN QUALITY FACTORS, Q, AND FLUENCE PER UNIT DOSE

EQUIVALENT FOR MONOENERGETIC NEUTRONS

|  | Neutron Energy (MeV) | Quality Factora (Q) | Fluence per Unit Dose Equivalentb (neutrons cm-2 rem-1) | Fluence per Unit Dose Equivalentb (neutrons cm-2 Sv-1) |
| --- | --- | --- | --- | --- |
| (thermal) | 2.5E–8 | 2 | 980E+6 | 980E+8 |
|  | 1E–7 | 2 | 980E+6 | 980E+8 |
|  | 1E–6 | 2 | 810E+6 | 810E+8 |
|  | 1E–5 | 2 | 810E+6 | 810E+8 |
|  | 1E–4 | 2 | 840E+6 | 840E+8 |
|  | 1E–3 | 2 | 980E+6 | 980E+8 |
|  | 1E–2 | 2.5 | 1010E+6 | 1010E+8 |
|  | 1E–1 | 7.5 | 170E+6 | 170E+8 |
|  | 5E–1 | 11 | 39E+6 | 39E+8 |
|  | 1 | 11 | 27E+6 | 27E+8 |
|  | 2.5 | 9 | 29E+6 | 29E+8 |
|  | 5 | 8 | 23E+6 | 23E+8 |
|  | 7 | 7 | 24E+6 | 24E+8 |
|  | 10 | 6.5 | 24E+6 | 24E+8 |
|  | 14 | 7.5 | 17E+6 | 17E+8 |
|  | 20 | 8 | 16E+6 | 16E+8 |
|  | 40 | 7 | 14E+6 | 14E+8 |
|  | 60 | 5.5 | 16E+6 | 16E+8 |
|  | 1E+2 | 4 | 20E+6 | 20E+8 |
|  | 2E+2 | 3.5 | 19E+6 | 19E+8 |
|  | 3E+2 | 3.5 | 16E+6 | 16E+8 |
|  | 4E+2 | 3.5 | 14E+6 | 14E+8 |

a. Value of quality factor (Q) at the point where the dose equivalent is maximum in a 30-centimeter diameter cylinder tissue-equivalent phantom.

b. Monoenergetic neutrons incident normally on a 30-centimeter diameter cylinder tissue-equivalent phantom.

b.  The department may, by rule, regulation, or order, impose upon any licensee or registrant such requirements in addition to those established in these rules as it deems appropriate or necessary to minimize danger to public health and safety or property.

37.5(2) Reserved.

[ARC 0387D, IAB 6/24/26, effective 7/29/26]