567—101.411(455B) Closure requirements. Unless otherwise specified within a permit, a solid waste transfer station shall be closed in accordance with the provisions of 567—100.10(455B,455D) and this rule. Closure shall not be official until the department has given written certification of the completion of the following activities.

101.411(1) Proper disposal of all solid waste and litter at the site.

101.411(2) Cleaning of all dumpsters, compactors, roll-off boxes, and other solid waste receptacles that will remain on site, including the rinsing of all surfaces that have come in contact with solid waste.

101.411(3) Cleaning the solid waste transfer station building, if applicable, including the rinsing of all surfaces that have come in contact with solid waste or washwater.

101.411(4) Cleaning of all solid waste collection and transport vehicles that will remain on site, including the rinsing of all surfaces that have come in contact with solid waste.

101.411(5) Removal and proper management of all washwater in the washwater management system.

101.411(6) Locking all doors, gates, entrances and exits.

101.411(7) Reporting of the completion of these activities to the local political jurisdictions, the department, and the department field office with jurisdiction over the solid waste transfer station.

[ARC 0482D, IAB 8/19/26, effective 9/23/26]