567—102.103(455B) Waste eligible for land application. Material must conform with the following to land applied pursuant to this division.

102.103(1) The material must either meet the definition of solid waste or be a sludge resulting from commercial or industrial wastewater treatment, water supply treatment, or air pollution control facility.

102.103(2) The waste shall not contain constituents in excess of the levels specified below measured on a dry weight basis.

| Constituents | Levels | Cumulative Loading Rate |  |
| --- | --- | --- | --- |
| Arsenic | 41 mg/kg | 41 kg/ha | 36 lb/ac |
| Cadmium | 39 mg/kg | 39 kg/ha | 34 lb/ac |
| Chromium | 1200 mg/kg | 3000 kg/ha | 2670 lb/ac |
| Copper | 1500 mg/kg | 1500 kg/ha | 1335 lb/ac |
| Lead | 300 mg/kg | 300 kg/ha | 267 lb/ac |
| Mercury | 17 mg/kg | 17 kg/ha | 15 lb/ac |
| Molybdenum | 75 mg/kg | 75 kg/ha | 66 lb/ac |
| Nickel | 420 mg/kg | 420 kg/ha | 373 lb/ac |
| Selenium | 36 mg/kg | 100 kg/ha | 89 lb/ac |
| Zinc | 2800 mg/kg | 2800 kg/ha | 2490 lb/ac |

102.103(3) If the waste has other toxic constituents, the toxic constituents shall not be in excess of levels where there is a threat to human, animal, or plant life as determined by the department.

102.103(4) The waste does not have a sodium absorption ratio in excess of levels where there is a threat to plant life. If high sodium absorption ratios are suspected, analytical testing may be required.

102.103(5) If the waste contains pathogens, the waste must be treated to reduce pathogen content by methods specified in 567—Chapter 67 prior to land application.

102.103(6) The waste shall not have direct process stream contact with or originate from a process that may release the following organic compounds.

a.  Petroleum products,

b.  Organic solvents,

c.  Pesticides,

d.  Pharmaceuticals,

e.  Polychlorinated biphenyls (PCBs).

102.103(7) The waste assimilates and would not be readily present in a visual analysis of a random sample collected two years following application.

[ARC 0483D, IAB 8/19/26, effective 9/23/26]