567—102.211(455B) Landfarm closure.

102.211(1) A landfarm shall be closed three years after the application of PCS unless otherwise authorized or required by this rule.

102.211(2) A landfarm may be closed if, at least six months after the application of PCS, documentation has been submitted and acknowledged in writing by the department that each landfarm plot has been tested as follows.

a.  One sample from each 10,000 ft2 (e.g., 100-foot × 100-foot area) of landfarm plot is analyzed pursuant to 102.205(2)“c”(1), “c”(2), and “c”(3). A minimum of one sample per landfarm plot shall be obtained. All samples shall be obtained from between the top two to six inches of soil.

b.  The results of the tests in 102.211(2)“a” demonstrate that petroleum constituent concentrations are less than 0.54 mg/kg for benzene, 42 mg/kg for toluene, 15 mg/kg for ethylbenzene, 3800 mg/kg for TEH-diesel and 0.02 mg/kg for MTBE.

[ARC 0483D, IAB 8/19/26, effective 9/23/26]